Final Exam Report

1. Draw a 1-bit full adder (FA), 2-input NAND, and 2-input AND cells using MAGIC, make sure it passes all DRCs, extract the circuit level netlist, and simulate its behaviour using SPICE. (2 points each: 6 points total)

Full Adder:

The NGspice results for inputs:

- A=1, B=1, cin=1 result: Sum= 1 , cout=1

Nand Gate:

A=1 B=1

And Gate:

A=1 B=1

2. Use hierarchical design flow to instantiate cells from Q1 to create CSA-W (white) and CSA-G (grey) cells shown on slide-3, make sure it passes all DRCs, extract the circuit level netlist, and simulate their behaviour using SPICE. (2 points each: 4 points total)

CSA-White:

X-1 Y-1  And gate Output is high, so A=1 B=1  C=1 sum=1 Carry=1

CSA- Grey:

X-1 Y-1  Nand gate Output is low, so A=0 B=1  C=1 sum=0 Carry=1

3Q) Use hierarchical design flow to instantiate CSA-W (white), CSA-G (grey), and FA cells from Q2 and flip-flops from DE-5, to create a 2-bit Baugh-Wooley signed multiplier shown on slide-4. Make sure it passes all DRCs, extract the circuit level netlist. (10 points)

The top and right rail is VDD and bottom and left rail is Gnd

Here the row left FA input A is connected to VDD.

Case1: 0 x 0 = 0000

X1, X0= 0 0

Y1, Y0= 0 0

S3, S2, S1, S0= 0 0 0 0 ( in the plot Sum3 is always high because A is connected to Vdd)

Case2: 1 x 1=1(decimal)

X1, X0= 0 1

Y1, Y0= 0 1

S3, S2, S1, S0= 0 0 0 1  ( S0 is white in colour) at t=3ns

Case3: 1 x -1= 0011( -1 in decimal)

X1, X0= 0 1

Y1, Y0= 1 1(-1 in decimal)

S3, S2, S1, S0= 0 0 1 1(-1 in decimal) (S0-white in colour and S1-orange) at t=3ns

Case4: -2 x -1= 0010( 2 in decimal)

X1, X0= 1 0 (-2 in decimal)

Y1, Y0= 1 1(-1 in decimal)

S3, S2, S1, S0= 0 0 1 0( 2 in decimal) ( S1-orange) at t=3ns

Q5) Find the maximum clock frequency using SPICE. (2 points).

The Critical path is marked with a dotted line where, the delay is from two CSA-G’s and two FA’s.

The max clock frequency is 0.24 Ghz.